EXAMPLE DOCUMENT. Stravane Fabrication (Pty) Ltd is a fictitious company. Every company, person, address, date, figure and reference number in this example is made up; only the Care Net letterhead and any public emergency number are real. For guidance only; not a document to sign or rely on.

Example for a Health and Safety File, Section G: Permits and controls

# Hot Work Permit to Work

Permit HW 0142. Welding repair to a steel storage tank support, Stravane Fabrication, Vanderbijlpark

| About this example | Detail |
| --- | --- |
| Company in the example | Stravane Fabrication (Pty) Ltd, a fictitious company |
| Where it is filed | Section G: Permits and controls |
| Published by | Care Net Consultants, as guidance for companies building a Health and Safety File |

## How to use this example

This example shows the form and content Care Net recommends for this document in a Health and Safety File. Adapt it to your company, your sites and your appointments; your registered Health and Safety practitioner confirms it before your File is released. Not legal advice.

## Contents

1. Permit details

2. Hazards identified

3. Isolation and preparation

4. Gas test

5. Fire watch

6. Personal protective equipment

7. Sign on

8. Sign off and hand back

# Hot Work Permit to Work

Permit HW 0142. Welding repair to a steel storage tank support, Stravane Fabrication, Vanderbijlpark

## 1. Permit details

| Item | Detail |
| --- | --- |
| Permit number | HW 0142 (from the numbered permit book; copy stays in the book) |
| Date and valid period | Tuesday 12 May 2026, 08:00 to 16:00. Not valid after 16:00 or if conditions change |
| Location | Bay 3, diesel storage tank support frame, Vanderbijlpark plant (fictitious) |
| Work | Arc welding and grinding to repair a cracked support leg on the empty diesel storage tank frame |
| Permit issuer | Johan van Wyk, Plant Engineer, authorised in writing to issue hot work permits |
| Permit receiver (person in charge of the work) | Sipho Dlamini, Boilermaker Foreman |
| Work team | Sipho Dlamini (welder), Thabo Nkosi (assistant) |
| Fire watch after work stops | For the period the permit procedure and the risk assessment set; the issuer wrote 14:40 to 15:40 on this permit |

## 2. Hazards identified

Fire or explosion from diesel vapour or residue near the tank; sparks and hot slag reaching combustible material; arc flash, fumes and hot surfaces; electrical shock from the welding set.

## 3. Isolation and preparation

| Control | Done | Checked by |
| --- | --- | --- |
| Tank drained, fill and outlet valves closed, locked and tagged (lock numbers SFB 21 and SFB 22) | Yes, 07:20 | J. van Wyk |
| Transfer pump electrically isolated at the board, locked and tagged (lock SFB 23); start tested and failed to start | Yes, 07:25 | J. van Wyk |
| Combustible material removed from the work area; drains and openings covered with fire blankets | Yes, 07:35 | S. Dlamini |
| Welding screens erected; area barricaded and hot work signs posted | Yes, 07:40 | S. Dlamini |
| Welding set, cables and earth clamp inspected; register entry current (Section F) | Yes, 07:45 | S. Dlamini |
| Two 9 kg dry chemical powder extinguishers and a charged hose reel at the work area | Yes, 07:45 | M. Molefe |

## 4. Gas test

Required, because the work is next to a tank that held diesel. Tested with a calibrated flammable gas detector (calibration record on file) at the tank openings, under the frame and at the drains.

| Time | Flammable gas reading | Result against the permit procedure | Tested by |
| --- | --- | --- | --- |
| 07:50 | 0% of the lower explosive limit | Acceptable; work may start | J. van Wyk |
| 11:30 | 0% of the lower explosive limit | Acceptable; work may continue | J. van Wyk |

## 5. Fire watch

Fire watch: Mpho Molefe, trained in the use of the extinguishers, stayed at the work area for the whole job with no other duties, and remained after the work stopped for the period the issuer wrote in part 1. Fire watch started 08:00 and ended 15:40 after a final check found no smouldering or hot spots.

## 6. Personal protective equipment

Welding helmet with the correct filter, leather gloves, apron and spats, safety boots, flame resistant overalls, hearing protection while grinding, and local fume extraction at the weld.

## 7. Sign on

The issuer and receiver confirm that the controls in parts 3 to 6 are in place and that the work team was briefed on the hazards and controls.

| Name | Role on this permit | Date and time | Signature |
| --- | --- | --- | --- |
| Johan van Wyk | Permit issuer | 12/05/2026 at 07:55 | Signed |
| Sipho Dlamini | Permit receiver | 12/05/2026 at 07:55 | Signed |
| Mpho Molefe | Fire watch | 12/05/2026 at 07:55 | Signed |
| Thabo Nkosi | Work team member (briefed) | 12/05/2026 at 07:55 | Signed |

## 8. Sign off and hand back

| Step | Time | By |
| --- | --- | --- |
| Work complete; team and tools withdrawn; area left clean | 14:40 | S. Dlamini |
| Final fire watch check: no smouldering, hot spots or damage | 15:40 | M. Molefe |
| Locks SFB 21, 22 and 23 removed; pump and valves returned to service; permit cancelled | 15:50 | J. van Wyk |

______________________________

Permit receiver (work complete; team and tools withdrawn)

Sipho Dlamini, Date: 12/05/2026 at 14:40

______________________________

Permit issuer (permit closed)

Johan van Wyk, Date: 12/05/2026 at 15:50

End of the example. The company, people and records in it are fictitious; the cover page explains how to use it.