EXAMPLE DOCUMENT. Neddikkie Artisan Bakeries (Pty) Ltd is a fictitious company. Every company, person, address, date, figure and reference number in this example is made up; only the Care Net letterhead and any public emergency number are real. For guidance only; not a document to sign or rely on.

Example for a Health and Safety File, Section C: Risk management

# Baseline Risk Assessment (Extract)

Hazard register for the Kya Sand bakery, signed 16 February 2026

| About this example | Detail |
| --- | --- |
| Company in the example | Neddikkie Artisan Bakeries (Pty) Ltd, a fictitious company |
| Where it is filed | Section C: Risk management |
| Published by | Care Net Consultants, as guidance for companies building a Health and Safety File |

## How to use this example

This example shows the form and content Care Net recommends for this document in a Health and Safety File. Adapt it to your company, your sites and your appointments; your registered Health and Safety practitioner confirms it before your File is released. Not legal advice.

## Contents

1. Scope, team and method

2. Hazard register

3. Employee consultation

4. Review

# Baseline Risk Assessment (Extract)

Hazard register for the Kya Sand bakery, signed 16 February 2026

## 1. Scope, team and method

| Item | Detail |
| --- | --- |
| Site and activities | Bakery at 41 Bernie Street, Kya Sand, Randburg: flour receiving and storage, mixing, dividing and moulding, baking, packing, cleaning, dispatch and maintenance. 88 employees on two shifts. |
| Risk assessor | Karabo Mokoena, registered health and safety practitioner, appointed in writing as risk assessor on 12 January 2026 (appointment and registration filed in Section B) |
| Team | Deon Swanepoel, Production Manager; Gift Mathebula, Maintenance Foreman; Zanele Khumalo, health and safety representative, mixing and baking; Ayesha Patel, health and safety representative, packing and dispatch |
| Assessment dates | 2 to 13 February 2026, on both shifts |
| Method | Walk through of every area and task, talks with the people doing the work, and a review of the incident register for 2025. Each risk is rated as likelihood (1 rare to 5 almost certain) times severity (1 first aid case to 5 fatality or permanent disability): 1 to 6 low, 8 to 12 medium, 15 to 25 high. Controls are chosen in the order elimination, substitution, engineering, administrative, then protective equipment. |

## 2. Hazard register

| Hazard and possible harm | Risk (L x S) | Controls, in the order of the hierarchy | Residual risk | Owner |
| --- | --- | --- | --- | --- |
| 1. Flour dust in mixing and when tipping bags: breathing problems and skin irritation | 16 High (4 x 4) | Substitution: bulk flour silo feed replaces hand tipping on mixers 1 and 2. Engineering: local exhaust ventilation at the tipping station. Administrative: no dry sweeping; vacuum cleaning; exposure monitoring by an approved inspection authority. Protective equipment: respirators fit tested for bag tipping. | 8 Medium | Deon Swanepoel, review 16/02/2027 |
| 2. Hot ovens, trays and racks: burns | 9 Medium (3 x 3) | Engineering: oven door guards and rack stops. Administrative: hot surface signs; safe work procedure for loading ovens. Protective equipment: heat resistant gloves and sleeves. | 4 Low | Deon Swanepoel, review 16/02/2027 |
| 3. Moving parts of mixers and dividers during cleaning: crushing and amputation | 10 Medium (2 x 5) | Engineering: interlocked guards on every mixer and divider. Administrative: lockout before cleaning or clearing a blockage; only trained operators; daily guard check by the supervisor. | 5 Low | Gift Mathebula, review 16/02/2027 |
| 4. Forklifts and pedestrians at the dispatch dock: being struck | 15 High (3 x 5) | Engineering: barriers and marked walkways that keep pedestrians out of the loading bay; blue spot lights on forklifts. Administrative: no pedestrians in the loading bay while loading is under way; trained and authorised operators only; daily pre use checks; speed limit. | 5 Low | Deon Swanepoel, review 16/02/2027 |
| 5. Lifting 25 kg flour bags and full trays: back and shoulder injuries | 12 Medium (4 x 3) | Engineering: bag lift assist at the small batch mixer; trolleys for trays. Administrative: team lifts; manual handling training; job rotation every two hours. | 6 Low | Deon Swanepoel, review 16/02/2027 |
| 6. Cleaning chemicals, including caustic oven cleaner: burns to skin and eyes | 12 Medium (3 x 4) | Substitution: milder degreaser for daily cleaning. Engineering: dosing unit for dilution. Administrative: safety data sheets at the chemical store; chemical handling training. Protective equipment: goggles, gloves and aprons; eyewash station. | 6 Low | Ayesha Patel, review 16/02/2027 |
| 7. Wet and floury floors: slips and falls | 8 Medium (4 x 2) | Engineering: slip resistant flooring in the wash area. Administrative: clean as you go; wet floor signs. Protective equipment: slip resistant safety shoes. | 4 Low | Deon Swanepoel, review 16/02/2027 |

## 3. Employee consultation

The draft register was discussed at the health and safety committee meeting of 11 February 2026 and at toolbox talks on both shifts on 12 and 13 February 2026, attended by 79 employees (registers filed in Section D). Employees asked for a second eyewash station at the wash bay and for job rotation on the packing line; both were added. The health and safety representatives walked the site with the risk assessor and signed below.

## 4. Review

This assessment is reviewed every year, and sooner after an incident, a new machine, a new substance, a change in a process or a request from the health and safety committee. Issue based and task based risk assessments are filed with it in Section C.

______________________________

Risk assessor

Karabo Mokoena, Date: 16/02/2026

______________________________

Health and safety representative

Zanele Khumalo, Date: 16/02/2026

______________________________

Approved: Managing Director

Rudi Venter, Date: 16/02/2026

End of the example. The company, people and records in it are fictitious; the cover page explains how to use it.